TAKEOFF

	Normal Takeoff – FLAPS 0°
	Short Field Takeoff – FLAPS 0°

	
	Hold Brakes

	Power 2000 RPM – Check Engine Gauges
	Power 2000 RPM – Check Engine Gauges

	
	Release Brakes

	Rotate VR 75K – Takeoff VLOF 80K
	Rotate VR 70K

	
	Climb 75K to 50'

	Climb BLUELINE – 88K
	Climb BLUELINE – 88K

	No Runway – Positive Rate – GEAR UP
	No Runway – Positive Rate – GEAR UP

	500' AGL – 100K
	500' AGL – 100K

	1000' AGL – 24" / 2500 RPM – Takeoff Checklist
	1000' AGL – 24" / 2500 RPM – Takeoff Checklist


APPROACH

	Normal Approach (1 or 2 Engine)
	Short Field Approach

	Approach Checklist 
	Approach Checklist 

	Downwind 100K (17")
	Downwind 100K (17")

	GEAR DOWN / FLAPS 25°
	GEAR DOWN / FLAPS 25°

	Descend BLUELINE – 88K (15")
	Descend BLUELINE – 88K (15")

	“GEAR DOWN – CHECKLIST”
	“GEAR DOWN – CHECKLIST”

	Base – “BLUELINE GUMP”
	Base – “BLUELINE GUMP”

	
	600' AGL / FLAPS 40 / SPEED 75K

	400' AGL – “GEAR DOWN STABILIZED”
	400' AGL – “GEAR DOWN STABILIZED”

	Power Slow RETARD over RUNWAY END
	Power Slow RETARD in FLARE

	Touchdown 75K
	Touchdown – FLAPS UP / MAXIMUM BRAKING


INSTRUMENT APPROACHES

	ILS Approach (1 or 2 Engine)
	Non-Precision Approach (1 or 2 Engine)
	Circle to Land (2 Engine ONLY)

	Approach Checklist
	Approach Checklist
	

	100K to Intercept (17")
	100K Procedure Turn (17")
	

	G/S Int. – GEAR DOWN / FLAPS 25°
	2E – 2Miles FAF GEAR DOWN / FLAPS 25°

1E – FAF GEAR DOWN / FLAPS 0°
	FAF – FLAPS 0°

	G/S Intercept – “NO FLAGS”

	FAF – “NO FLAGS”
	

	G/S Descent – BLUELINE - 88K (2E - 15" / 1E - 20")
	FAF Descent – 88K (2E - 13" / 1E - 15")
	

	“GEAR DOWN – CHECKLIST”
	“GEAR DOWN – CHECKLIST”
	

	1000' AGL “BLUELINE GUMP”
	1000' AGL “BLUELINE GUMP”
	

	400' AGL “GEAR DOWN – STABILIZED”
	100' ABOVE MINS “100 TO GO”
	

	100' ABOVE MINS “100 TO GO”
	MDA – Level (2E - 20" / 1E – FULL)
	

	
	1E - Leaving MDA – FLAPS 25°
	Turning Final, Leaving MDA – FLAPS 25°

	
	400' AGL “GEAR DOWN – STABILIZED”
	

	Power Slow RETARD over RUNWAY END
	Power Slow RETARD over RUNWAY END
	

	Touchdown 75K
	Touchdown 75K
	


MISSED APPROACH

	Full Power – PITCH 88K
	
	

	Slow FLAPS 0°
	
	HOLDING SPEED

	Positive Rate – GEAR UP
	
	100K

	500' AGL – 100K
	
	

	1000' AGL - 24" / 2500 RPM – Takeoff Checklist
	
	


	STEEP TURNS
	
	VMC DEMO

	SPEED 120K - 20" / 2300 RPM
	
	Clearing Turns – 100K (17")

	50° BANK – POWER 23" - TRIM
	
	LEFT Throttle – IDLE

	
	
	RIGHT Throttle – FULL POWER

	
	
	5° BANK

	
	
	PITCH 1° / SEC – LIMIT RUDDER @ 77K

	
	
	LOSS OF DIRECTIONAL CONTROL or

STALL WARNING

	
	
	PITCH DOWN – POWER TO IDLE

	
	
	Slow POWER to FULL – 82K and then 88K


STALLS and SLOW FLIGHT

	Slow Flight
	Stall – Power Off
	Stall – Power On

	Clearing Turns – 100K (17")
	Clearing Turns – 100K (17")
	Clearing Turns – 100K (17")

	GEAR DOWN / FLAPS 40°
	GEAR DOWN / FLAPS 40°
	

	GEAR DOWN Checklist
	GEAR DOWN Checklist
	

	LEVEL FLIGHT – 70K
	DESCENT – 80K
	POWER 13" / SPEED 80K

	
	POWER to IDLE – Pitch to BUFFET
	POWER 18" / Pitch to BUFFET

	CRUISE POWER
	PITCH DOWN – FULL POWER
	PITCH DOWN – FULL POWER

	FLAPS 0° / SPEED 82K
	FLAPS 0° / SPEED 82K
	SPEED 82K

	Positive Rate – GEAR UP
	Positive Rate – GEAR UP
	

	SPEED 88K
	SPEED 88K
	SPEED 88K


CONFIGURATION CHECKLIST

	Fuel Selectors – ON
	Mixtures – RICH

	Cowl Flaps – OPEN
	Prop – HIGH

	
	Fuel Pumps - ON


